## DL710 VME Interface

The DL710 VMEcard supports only the Standard Non-Privileged Data Access (A24) with address modifier AM=0x39. The data access is only in 32 bit longwords (D32).

Block transfers are not supported.

Two HEX-Switches on the front panel determine the individual card address.

Only Bit 3 of the low order switch (L) and Bit 0..3 of the high order switch (H) are relevant!

The remaining 19 bits are used for the internal address space. Please note that Bit 1 and 0 are always ‘0’ for longword access!

|  | Card address |  |  |  |  | Internal address space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
|  | H3 | H 2 | H 1 | H 0 | L 3 | A | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | 0 | 0 |

The Internal address space is further divided by Bit 18 (A) into

A= 0: CBus L_address space

A= 1: Control Space with several special addresses (see Table 1).

| Vme Address | Name | Operation | Description |
| --- | --- | --- | --- |
| 0x000000 0x03FFFF | CBus L_address space | write/read |  |
| 0x040000 | FPGA ID | read | value= 0x00010205 |
| 0x040004 | Control register | write | Bit 0 = Reset |
| 0x040008 | CBus H_address | write/read | 16 Bit |
| 0x04000C 0xFFFFFF | Reserved |  |  |

Table 1:	DL710 memory map

All functional modules are located in an internal 32 bit address space with full 32 bit data (CBus).

The CBus address is combined on every access from an internal (preloaded) register (CBus H_address ) and the current VME address (CBus L_address) and addresses longwords!

CBus data is the current 32 Bit from VME.

Figure 1 shows how the module address is built with the internal register and the VME address.

CBus H_address register

VME address

Bit 0

Bit 7

Bit 15

Bit 23

Bit 0

Bit 7

Bit 15

Bit 23

Bit 31

Bit 0

Bit 7

Bit 15

Bit 23

Bit 31

CBus address

Figure 1:	CBus address.

Presently 4 Counter modules are implemented in the FPGA.

The CBus address space is organized as follows:

| not used | Type (0x45) | Module # | Subaddress |
| --- | --- | --- | --- |
| A24 | A16 | A8 | A0 |

To address a specific Counter module, you have to preload in an initial access the CBus H_address register and load the upper Cbus address bits. In this case the value „0x45“ has to be loaded (see example below).

Module # and Subaddress are taken from the actual VME address (shifted for longword access!).

The Module Subaddress space for one Counter module is shown in Table 2.

| Module Subaddress | Name | Operation | Description |
| --- | --- | --- | --- |
| 0x00 | Control register | write/read | 32 Bits; Bit 0: set the timer to 0 |
| 0x01 | Divider | write/read | Bit 0 to 7: set the divider for the 1 us clock; Value=0: Inhibit Timer |
| 0x02 | Timer value | read | Bit 0 to 31. Read the timer value |

Table 2:	Module subaddress memory map

Example:

The following VME operations must be done to control the timer 1 and 2  the HEX switch address is 0xFF.

Set the Hi module address to timer address:

WriteVmeA24D32(0xFC0008, 0x00000045);

Set the timer divider of the timer 1 to  5 us:

WriteVmeA24D32(0xF80000 + (0x0101<<2), 0x00000005);

Set the timer divider of the timer  to 10 us:

WriteVmeA24D32(0xF80000 + (0x0201<<2), 0x0000000A);

Read the value of the timer 1 and 2:

ReadVmeA24D32(0xF80000 + (0x0102<<2), &dwData);

ReadVmeA24D32(0xF80000 + (0x0202<<2), &dwData);